| Op. Code | Op. Description | Time (s) Trial 1 | Time (s) Trial 2 | Time (s) Trial 3 | Time (s) Trial 4 | Average | Predecessors |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Place Soleplate and heating element as a base foundation of the iron | 3 | 2 | 2 | 3 | 2,5 | - |
| B | Fit the Soleplate perimeter seal onto the heating element | 2 | 3 | 2 | 3 | 2,5 | A |
| C | Install the Tightening washer to heating unit | 7 | 4 | 3 | 6 | 5 | B |
| D | Fit the soleplate cover onto the heating element unit | 15 | 19 | 16 | 13 | 15,75 | C |
| E | Fasten Self-tapping screw — soleplate BIG screw to secure the base | 20 | 18 | 19 | 22 | 19,75 | D |
| F | Drive the Self-tapping screw — soleplate SMALL screw to finish the base | 43 | 47 | 46 | 40 | 44 | D |
| G | Place the Water tank O-ring onto the water tank | 20 | 16 | 23 | 21 | 20 | - |
| H | Secure the Intel knob water to steam tube | 8 | 6 | 6 | 10 | 7,5 | - |
| I | Insert steam tube with steam trigger | 15 | 14 | 17 | 11 | 14,25 | H |
| J | Insert water tube to water trigger | 16 | 15 | 13 | 17 | 15,25 | - |
| K | Attach water trigger to body | 4 | 7 | 5 | 8 | 6 | J |
| L | Attach steam trigger to body | 4 | 1 | 7 | 1 | 3,25 | IH |
| M | Connect spray nozzle with water tube and attach it to body | 15 | 16 | 11 | 13 | 13,75 | KJ |
| N | Insert Temperature control dial into slot on the body | 10 | 11 | 7 | 13 | 10,25 | - |
| O | Plug the Thermostats connector into the body | 2 | 6 | 1 | 6 | 3,75 | N |
| P | Mount the main tank onto the base | 11 | 14 | 8 | 15 | 12 | OM |
| Q | Tighten the Handle screw | 28 | 30 | 28 | 29 | 28,75 | P |
| R | Tighten Body screw to fix the body | 38 | 41 | 41 | 42 | 40,5 | P |
| S | Connect power cord to heating assembly | 40 | 37 | 41 | 39 | 39,25 | R |
| T | Plug LED pilot indicator into body | 7 | 9 | 3 | 4 | 5,75 | R |
| U | Attach the LED protector over LED pilot indicator | 5 | 6 | 6 | 9 | 6,5 | T |
| V | Fit 360° cord swivel contact into the back of body housing | 5 | 4 | 4 | 8 | 5,25 | U |
| W | Snap the Water cap onto handle | 6 | 4 | 4 | 5 | 4,75 | - |
| X | Snap the Steam cap onto handle | 6 | 7 | 2 | 2 | 4,25 | - |
| Y | Install the Cable keeper onto the back of the iron body | 7 | 9 | 5 | 6 | 6,75 | V |
| Z | Fasten the Cable keeper screw to secure the cable keeper | 35 | 33 | 37 | 36 | 35,25 | Y |
| AA | Place the Fill cap O-ring onto the water fill of body | 6 | 5 | 9 | 2 | 5,5 | P |
| AB | Install the Handle over the body, ensuring it locks with the screws and swivel contact | 60 | 56 | 64 | 59 | 59,75 | AAWXUV |
| AC | Insert self-clean valve into handle | 9 | 6 | 6 | 13 | 8,5 | AB |
| AD | Drive the Handle-body fix screw to secure the handle to the body | 13 | 9 | 12 | 16 | 12,5 | AB |